    - Calculate the total uniformly distributed roof dead load in psf of horizontal plan area for a sloped roof with the design parameters given below.

- 2x8 rafters at 24” on centers

- Asphalt shingles on ½” plywood sheathing

- 6” insulation (fiberglass)

- Suspended Ceiling

- Roof slope: 6-in-12

- Mechanical & Electrical (i.e. ducts, plumbing etc) = 5 psf

Solution:

2x8 rafters at 24” on-centers  		= 1.2 psf

Asphalt shingles (assume ¼” shingles)		= 2.0 psf

½” plywood sheathing = (4 x 0.4 psf/1/8” plywood) = 1.6 psf

6” insulation (fiberglass) = 6 x 1.1 psf/in.		= 6.6 psf

Suspended Ceiling 		= 2.0 psf

Mechanical & Electrical (i.e. ducts, plumbing etc) 	= 5.0 psf

Total roof dead load, D (psf of sloped roof area) = 18.4 psf

The total dead load in psf of horizontal plan area will be:

,  psf of horizontal plan area

=	  20.6 psf of horizontal plan area

  - Given the following design parameters for a sloped roof, calculate the uniform total load 	and the maximum shear and moment on the rafter. Calculate the horizontal thrust on the 	exterior wall if rafters are used.

- Roof dead load, D	= 20 psf (of sloped roof area)

- Roof snow load, S 	= 40 psf (of horizontal plan area)

- Horizontal projected length of rafter, L2  =	14 ft

- Roof slope: 4-in-12

- Rafter or Truss spacing =  4’ 0

Solutions:

Sloped length of rafter, L1   =

Using the load combinations in section 2.1, the total load in psf of horizontal plan area will be:

+  (Lr  or S or  R),  psf of horizontal plan area

+  40 psf

=	61.1 psf of horizontal plan area

The total load in pounds per horizontal linear foot (Ib/ft) is given as,

wTL	(Ib/ft)	=	wTL (psf) x 	Tributary width (TW) or Spacing of rafters

=	61.1 psf (4 ft)  = 244.4 lb/ft.

h = (4/12) (14 ft) = 4.67 ft

The horizontal thrust H is,

=   = 5129 Ib.

The collar or ceiling ties must be designed to resist this horizontal thrust.

L2 = 14’

The maximum shear force in the rafter is,

=  = 1711 Ib

The maximum moment in the rafter is,

=  = 5989 ft-Ib = 5.9 ft-kip

  - Determine the tributary widths and tributary areas of the joists, beams, girders and columns in the panelized roof framing plan shown below. Assuming a roof dead load of 20 psf and an essentially flat roof with a roof slope of ¼” per foot for drainage, determine the following loads using the IBC load combinations. Neglect the rain load, R and assume the snow load, S is zero:

- The uniform total load on the typical roof joist in Ib/ft

- The uniform total load on the typical roof girder in Ib/ft

- The total axial load on the typical interior column, in Ib.

- The total axial load on the typical perimeter column, in Ib

.

Solution:

The solution is presented in a tabular format as shown below:

Tributary Widths and Tributary Areas of Joists, Beams and Columns

| Structural Member | Tributary Width (TW) | Tributary Area (TA) |
| --- | --- | --- |
| Purlin |  | 10’ x 20’ = 200 ft2 |
| Glulam girder |  | 20’ x 60’ = 1200 ft2 |
| Typical interior Column |  | 1200 ft2 |
| Typical perimeter Column |  | 600 ft2 |

Since the snow and rain load are both zero, the roof live load, Lr will be critical.

With a roof slope of ¼” per foot, the number of inches of rise per foot, F = ¼ = 0.25

Purlin:

The tributary width TW = 10 ft and the tributary area, TA = 200 ft2 < 200 ft2

From section 2.5.1, we obtain:R1 = 1.0 and R2 = 1.0,

Using equation 2-4 gives the roof live load, Lr = 20 x 1 x 1 = 20 psf

The total loads are calculated as follows:

wTL (psf) = (D + Lr) = 20 + 20 = 40 psf

wTL (Ib/ft) = wTL (psf) x tributary width (TW) = 40 psf x 10 ft = 400 Ib/ft

Glulam Girder:

The tributary width, TW = 20 ft and the tributary  Area, TA = 1200 ft2

Thus, TA > 600, and from section 2.4, we obtain:

R1 =  0.6, and

R2 = 1.0

Using equation 2-4 gives the roof live load, Lr = 20 x 0.6 x 1 = 12 psf

The total loads are calculated as follows:

wTL (psf) = (D + Lr) = 20 + 12 = 32 psf

wTL (Ib/ft) = wTL (psf) x tributary width (TW) = 32 psf x 20 ft = 640 Ib/ft

Typical Interior Column:

The tributary tributary area of the typical interior column, TA = 1200 ft2

Thus, TA > 600, and from section 2.4, we obtain:

R1 =  0.6, and

R2 = 1.0

Using equation 2-4 gives the roof live load, Lr = 20 x 0.6 x 1 = 12 psf

The total loads are calculated as follows:

wTL (psf) = (D + Lr) = 20 + 12 = 32 psf

The Column Axial Load, P = 32 psf x 1200 ft2 = 38, 400 Ib  = 38.4 kips

Typical Perimeter Column:

The tributary tributary area of the typical perimeter column, TA = 600 ft2

Thus, from section 2.5.1, we obtain:

R1 =  0.6, and

R2 = 1.0

Using equation 2-4 gives the roof live load, Lr = 20 x 0.6 x 1 = 12 psf

The total loads are calculated as follows:

wTL (psf) = (D + Lr) = 20 + 12 = 32 psf

The Column Axial Load, P = 32 psf x 600 ft2 = 19, 200 Ib  = 19.2 kips

  - A building has sloped roof rafters (5:12 slope) spaced at 2’ 0” on centers and is located in Hartford, Connecticut. The roof dead load is 22 psf of sloped area. Assume a fully exposed roof with terrain category “C”, and use the ground snow load from the IBC or ASCE 7 snow map

(a) Calculate the total uniform load in lb/ft on a horizontal plane using the IBC.

(b) Calculate the maximum shear and moment in the roof rafter.

Solution:

The roof slope,  for this building is 22.6,

Roof Live Load, Lr:

From Section 2.4, the roof slope factor is obtained as,

F = 5		 R2 = 1.2 – 0.05 (5) = 0.95

Assume the tributary area (TA) of the rafter < 200 ft2,		 R1 = 1.0

The roof live load will be,

Lr = 20R1R2 = 20(1.0)(0.95) = 19 psf

Snow Load:

Using IBC Figure 1608.2 or ASCE 7 Figure 7-1, the ground snow load, Pg  for Hartford, Connecticut is 30 psf.

Assuming a building with a warm roof and fully exposed, and a building site with terrain category “C”, we obtain the coefficients as follows:

Exposure coefficient,  Ce = 0.9  (ASCE 7 Table 7-2)

The thermal factor, Ct = 1.0 (ASCE 7 Table 7-3)

The Importance Factor, I = 1.0 ASCE Table 7-4

The slope factor, Cs = 1.0 (ASCE Figure 7-2 with roof slope,  = 22.6 and a warm roof)

The flat roof snow load, 	Pf = 0.7 Ce Ct I Pg   = 0.7 x 0.9 x 1.0 x 1.0 x 30 = 18.9 psf

Minimum flat roof snow load, Pm = 20Is = 20 (1.0) = 20 psf (governs)

Thus, the design roof snow load, 	Ps = Cs Pf  = 1.0 x 20 = 20 psf

Therefore, the snow load, S = 20 psf

The total load in psf of horizontal plan area is given as,

+  (Lr or S or R) ,  	psf of horizontal plan area

Since the roof live load, Lr (18 psf) is smaller that the snow load, S (20 psf), the snow load is more critical and will be used in calculating the total roof load.

+  20 psf

=	43.83 psf of horizontal plan area

The total load in pounds per horizontal linear foot (Ib/ft) is given as,

wTL	(Ib/ft)	=	wTL (psf) x 	Tributary width (TW) or Spacing of rafters

=	43.83 psf (2 ft)  = 87.7 lb/ft.

Assume L2 = 14’

The maximum moment in the rafter is,

=  = 2149 ft-Ib = 2.15 ft-kip

  - A 3-story building has columns spaced at 18 ft in both orthogonal directions, and is subjected to the roof and floor loads shown below. Using a column load summation table, calculate the cumulative axial loads on a typical interior column with and without live load reduction. Assume a roof slope of ¼” per foot for drainage.

Roof Loads:

Dead Load, Droof 	= 20 psf

Snow Load, S 	= 40 psf

2nd and 3rd  Floor Loads:

Dead Load, Dfloor 	=  40 psf

Floor Live Load, L = 50 psf

Solution:

At each level, the tributary area (TA) supported by a typical interior column is

18’ x 18’ =	324 ft2

Roof Live Load, Lr:

From section 2.4, the roof slope factor is obtained as,

F = ¼  = 0.25		 R2 = 1.0

Since the tributary area (TA) of the column = 324 ft2,    R1 = 1.2 – 0.001 (324) = 0.88

The roof live load will be,

Lr = 20R1R2 = 20(0.88)(1.0) = 17.6 psf  < Snow load, S = 40 psf

The governing load combination from Section 2.1.1 for calculating the column axial loads is D + L + (Lr or S or R). Since the snow load is greater than the roof live load, the critical load combination reduces to D + L + S.

The reduced or design floor live load for the 2nd and 3rd floors are calculated using the table below:

Reduced or Design Floor Live Load Calculation Table

| Member | Levels supported | AT (summation of floor tributary area) | KLL | Unreduced Floor live load, Lo (psf) | Live Load Reduction Factor 0.25 + 15/(KLL AT) | Design floor live load, L |
| --- | --- | --- | --- | --- | --- | --- |
| 3rd floor Column (i.e. column below roof) | Roof only | Floor live load reduction NOT applicable to roofs!!! | - | - | - | 40 psf (Snow load) |
| 2nd floor column (i.e. column below 3rd floor) | 1 floor + roof | 1 floor x 324 ft2 = 324 ft2 | 4 KLL AT = 1296 > 400 ft2  Live Load reduction allowed | 50 psf | 0.25 + 15/(4 x 324) = 0.67 | 0.67 x 50 = 33.5 psf ≥ 0.50 Lo = 25 psf |
| Ground or 1st floor column (i.e. column below 2nd floor) | 2 floors + roof | 2 floors x 324 ft2 = 648 ft2 | 4 KLL AT = 2592 > 400 ft2  Live Load reduction allowed | 50 psf | 0.25 + 15/(4 x 648) = 0.54 | 0.54 x 50 = 27 psf ≥ 0.40 Lo = 20 psf |

The column axial loads with and without floor live load reduction are calculated using the column load summation tables below:

Column Load Summation Table

| Level | Tributary area, (TA) (ft2 ) | Dead Load, D (psf) | Live Load, Lo (S or Lr or R on the roof) (psf) | Design Live Load Roof: S or Lr or R Floor: L (psf) | Unfactored total load at each level, ws1 Roof: D Floor: D + L (psf) | Unfactored total load at each level,ws2 Roof: D+0.75S Floor: D+0.75L (psf) | Unfactored Column Axial Load at each level, P = (TA)(ws1) or (TA)(ws2) (kips) | Cumulative Unfactored Axial Load, PD+L (kips) | Cumulative Unfactored Axial Load, PD+0.75L+0.75S (kips) | Maximum Cumulative Unfactored Axial Load, P (kips) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| With Floor Live Load Reduction |  |  |  |  |  |  |  |  |  |  |
| Roof | 324 | 20 | 40 | 40 | 20 | 50 | 6.5 or 16.2 | 6.5 | 16.2 | 16.2 |
| 3rd Flr | 324 | 40 | 50 | 33.5 | 73.5 | 65.1 | 23.8 or 21.1 | 30.3 | 37.3 | 37.3 |
| 2nd Flr | 324 | 40 | 50 | 27 | 67 | 60.3 | 21.7 or 19.5 | 52 | 56.8 | 56.8 |
| Without Floor Live Load Reduction |  |  |  |  |  |  |  |  |  |  |
| Roof | 324 | 20 | 40 | 40 | 20 | 50 | 6.5 or 16.2 | 6.5 | 16.2 | 16.2 |
| Third floor | 324 | 40 | 50 | 50 | 90 | 77.5 | 29.2 or 25.1 | 35.7 | 41.3 | 41.3 |
| Second floor | 324 | 40 | 50 | 50 | 90 | 77.5 | 29.2 or 25.1 | 64.9 | 66.4 | 66.4 |

  - A 2-story wood framed structure 36 ft x 75 ft in plan is shown below with the following given information. The floor to floor height is 10 ft and the truss bearing (or roof datum) elevation is at 20 ft and the truss ridge is 28 ft 4” above the ground floor level. The building is “enclosed” and located in Rochester, New York on a site with a category “C” exposure. Assuming the following additional design parameters, calculate:

Floor Dead Load = 	30 psf

Roof Dead Load = 	20 psf

Exterior Walls = 	10 psf

Snow Load (Pf) =	40 psf

Site Class = 		D

Importance (Ie)=	1.0

SS =			0.25%

S1 =			0.07%

R = 			6.5

- The total horizontal wind force on the main wind force resisting system (MWFRS) in both the transverse and longitudinal directions.

- The gross vertical wind uplift pressures and the net vertical wind uplift pressures on the roof (MWFRS) in both the transverse and longitudinal directions.

- The seismic base shear, V, in kips

- The lateral seismic load at each level in kips

Solution:

(a) Lateral Wind

Roof Slope: Run = 18’, Rise = 8’-4”,  = 25

Assuming a Category II building

V = 115mph (ASCE 7 Table 26.5-1A)

Wind Pressures (from ASCE 7, Figure 28.6-1):

Transverse ( = 25):

Horizontal				Vertical

Zone A: 26.3 psf			Zone E: -11.7 psf

Zone B: 4.2 psf			Zone F: -15.9 psf

Zone C: 19.1 psf			Zone G: -8.5 psf

Zone D: 4.3 psf			Zone H: -12.8 psf

Longitudinal: ( = 0):

Horizontal				Vertical

Zone A: 21 psf			Zone E: -25.2 psf

Zone B: N/A				Zone F: -14.3 psf

Zone C: 13.9 psf			Zone G: -17.5 psf

Zone D: N/A				Zone H: -11.1 psf

End Zone width:

a:	   0.1 x least horizontal dimension of building

   0.4 x mean roof height of the building and

  0.04 x least horizontal dimension of building

- 3 feet

a	 0.1 (36’) = 3.6’ (governs)

 (0.4)20'+28.33'2= 9.67’ 
 0.04 (36’) = 1.44’

 3 feet

Therefore the Edge Zone = 2a = 2 (3.6’) = 7.2’

Average horizontal pressures:

Transverse:

qavg = (end zone)(end zone pressure)+(bldg width−end zone)(interior zone pressure)(bldgwidth)

= 19.8 psf (Zones A, C)

= 4.3 psf (Zones B, D)

Longitudinal:

= 15.32 psf (Zones A, C)

Design wind pressures:

Height and exposure coefficient:

Mean roof height = 20'+28.33'2= 24.2’

 = 1.35 (ASCE 7 Figure 28.6-1, Exposure = C,  h  25’)

Transverse wind:

P = qavg

Pwall = (19.8 psf)(1.35) = 26.73 psf

Proof = (4.3 psf)(1.35) = 5.81 psf

Longitudinal wind:

Pwall = (15.32 psf)(1.35) = 20.7 psf

Total Wind Force:

Transverse wind:

= 43.7 kips (base shear, transverse)

Longitudinal wind:

= 18.0 kips (base shear, longitudinal)

(b) Wind Uplift

Average vertical pressures:

P = qavg

From Part (a), base uplift pressures:

Transverse:			Longitudinal:

Zone E: -11.7 psf		Zone E: -25.2 psf

Zone F: -15.9 psf		Zone F: -14.3 psf

Zone G: -8.5 psf		Zone G: -17.5 psf

Zone H: -12.8 psf		Zone H: -11.1 psf

Transverse:

= - 39,922 lb.

=  - 14.8 psf (gross uplift, transverse)

Longitudinal:

= - 56,097 lb.

= - 20.8 psf (gross uplift, longitudinal)

Net factored uplift (Longitudinal controls):

qnet = 0.9D+W

=(0.9)(20psf) + (-20.8psf) = -2.8psf (net uplift)

(c) Seismic base shear

Calculate “W” for each level

| Level | Area (ft.2) | Trib. Height (ft.) | Wt. Level (kip)* | Wt. Walls (kip) | Wtotal (kip) |
| --- | --- | --- | --- | --- | --- |
| Roof | 75’ x 36’ = 2700 ft.2 | (10’/2)= 5’ | (2700 ft2) x [20psf + (0.2x40psf)] = 75.6k | (5’) x (10psf) x (2) x (75’ + 36’) = 11.1k | 75.6k + 11.1k = 86.7k |
| 2nd | 75’ x 36’ = 2700 ft.2 | (10’/2) + (10’/2) = 10’ | (2700 ft2) x (30psf) = 81.0k | (10’) x (10psf) x (2) x (75’ + 36’) = 22.2k | 81.0k + 22.2k = 103.2k |
|  |  |  |  | W = 86.7k + 103.2k = 189.9k |  |

* Note:  Where the flat roof snow load, Pf, is greater than 30psf, then 20% of the flat roof snow load shall be included in “W” for the roof (ASCE 7 Section 12.14.8.1 )

Seismic Variables:

Fa = 	1.6 (ASCE 7 Table 11.4-1)

Fv = 	2.4 (ASCE 7 Table 11.4-2) )

SMS = 	Fa SS 	= (1.6) ( 0.25) = 0.40

SM1 = 	Fa S1 	= (2.4) ( 0.07) = 0.168

SDS = 	(2/3) SMS = (2/3) ( 0.40) = 0.267

SD1 = 	(2/3) SM1 = (2/3) ( 0.168) = 0.112

Base Shear:

V = F SDS WR

V = (1.1) (0.267) (189.9)(6.5)= 8.58k

(d) Seismic Forces at each level:

Fx = F SDS WxR

FR = (1.1) (0.267) (86.7)(6.5)= 3.92k

F2 = (1.1) (0.267) (103.2)(6.5)= 4.67k

....

2-7 (see framing plan and floor section)
a) Determine the floor dead load in PSF
b) Determine the service dead and live loads to J-1 and G-1 in PLF
c) Determine the maximum factored loads in PLF to J-1 and G-1
d) Determine the factored maximum moment and shear in J-1 and G-1
e) Determine the maximum service and factored load in kips to C-1

2-9.  w = 500plf; L1 = 20ft

Case 1: continuous over support

Case 2: hinged over support

Case 1 would have less deflection

Case 2 is easier to build; 40ft section might be hard to get or ship/handle on-site

2-10

a) List each truss member in a table (B1, B2, T1, T2, W1 to W6) and list the following: size, length, species, grade, density, weight).

b) Calculate the total weight of the truss using the table in (a).

c) Draw a free-body diagram of the truss and indicate the uniformly distributed loads to the top and bottom chords in pounds per lineal foot (plf) and indicate the supports.
d) Calculate the maximum possible reaction using the controlling load case Dead + Snow.

| Trib width = 2 ft. Top chord: wD = (2)(10) = 20plf wLr = (2)(20) = 40plf wS = (2)(30.8) = 61.6plf Bot. chord: wD = (2)(10) = 20plf wD = 20+20 = 40 plf wS = 61.6 plf Rmax = (20+20+61.6)(19.41)2=986 lb. |  |
| --- | --- |

d) What are the maximum compression loads to W2, W5, and W6 and what is the purpose of the single row of bracing at midpoint?

W2 – 860 lb

W5 – 899lb

W6 – 354 lb

The bracing limits the unbraced length of the members being braced and prevents buckling under compression.

2-11:
Given Loads:

| Uniform load, w D = 500plf L = 800plf S = 600plf Beam length = 25 ft. | Concentrated Load, P D = 11k S = 15k W = +12k or -12k E = +8k or - 8k |
| --- | --- |

Do the following:

a) Describe a practical framing scenario where these loads could all occur as shown.
b) Determine the maximum moment for each individual load effect (D, L, S, W, E)
c) Develop a spreadsheet to determine the worst-case bending moments for the code-required load combinations.

- Transfer beam that has loads transferred from the roof down to a floor level.

Load Combinations

Uniform Loads

Concentrated Loads

2-12 (see framing plan)
Assuming a roof dead load of 25 psf and a 25 degree roof slope, determine the following using the IBC factored load combinations. Neglect the rain load, R and assume the snow load, S is zero:

- Determine the tributary areas of B1, G1, C1, and W1

- The uniform dead and roof live load and the factored loads on B1 in PLF

- The uniform dead and roof live load on G1 and the factored loads in PLF
(Assume G1 is uniformly loaded)

- The total factored axial load on column C1, in kips

- The total factored uniform load on W1 in PLF (assume trib. length of 50 ft.)

2-13. A 3-story building has columns spaced at 25 ft in both orthogonal directions, and is subjected to the roof and floor loads shown below. Using a column load summation table, calculate the cumulative axial loads on a typical interior column.  Develop this table using a spreadsheet.  Submit a hard copy that is properly formatted with your HW and submit the XLS file by e-mail.

| Roof Loads: Dead, D = 20psf Snow, S = 45psf | 2nd & 3rd floor loads Dead, D = 60psf Live, L = 100psf |
| --- | --- |

All other loads are 0

2-14 Using only the loads shown and the weight of the concrete footing only (conc = 150pcf), determine the required square footing size, BxB using the appropriate load combination to keep the footing from overturning about point A (i.e. - either load combination 6 or 15 Chapter 2 of the text).   Loads shown are service level (Mw = 0.6W = 45k-ft)

Overturning Moment

Resisting Moment

ASD Load Comb

LRFD Load Comb

must be greater than 1.0

Use B=7.28ft for ASD and 7.67ft for LRFD

2-15.
Given:
Location - Massena, NY; elevation is less than 1000 feet
Total roof DL = 25psf
Ignore roof live load; consider load combination 1.2D+1.6S only
Use normal occupancy, temperature, and exposure conditions
Length of B-1, B-2 is 30 ft.

Find:
a) Flat roof snow load and sloped roof snow load
b) Sliding snow load
c) Determine the depth of the balanced snow load and the sliding snow load on B-1 and B-2
d) Draw a free-body diagram of B-1 showing the service dead and snow loads in PLF
e) Find the factored Moment and Shear in B-1.

2-16.
Given:
Location - Pottersville, NY; elevation is 1500 feet
Total roof DL = 20psf
Ignore roof live load; consider load combination 1.2D+1.6S only
Use normal occupancy, temperature, and exposure conditions

Find:
a) Flat roof snow load
b) Depth and width of the leeward drift and windward drifts; which one controls the design of J-1?
c) Determine the depth of the balanced snow load and controlling drift snow load
d) Draw a free-body diagram of J-1 showing the service dead and snow loads in PLF

Problem 2-16 Part (e)

Factored Reactions and Maximum Moment:

RA = 63.18 kips

RB = 46.85 kips

Mu, max = 1207.6 ft-kips (occurs at 51.55 ft from B)