Computer Program

Animation-Visualization Of Radiation Problems

c***********************************************************

c This MATLAB-based program contains three separate

c  animation-visualization radiation problems; namely:

c	I.	Line-Source; Gaussian pulse: Unbounded medium (tm_open)

c  	II.	Line-Source-Gaussian pulse: PEC square cylinder (tm_box)

c  	III.	E-Plane sectoral horn: unbounded medium (te_horn)

c  The objective is to allow the user to animate and then to visualize

c  radiation, as a function of time, of three different radiation problems.

c

c  	I.  	Line-Source; Gaussian pulse: unbounded medium (tm_open)

c  The first animation-visualization program is that of a line source excited by

c  a single Gaussian pulse radiating in an unbounded medium, using the

c  Finite-Difference Time-Domain (FDTD) method.  The unbounded medium

c  is simulated using a Berenger Perfectly-Matched Layer (PML) Absorbing

c  boundary Condition (ABC) in order to truncate the computational domain.

c  The MATLAB m-file  produces the FDTD solution of an infinite length line

c  source excited by a time-derivative Gaussian pulse in a 2-D TMz

c  computational domain.  The m-file produces a movie which is 37 frames

c  long by taking a picture of the computational domain every 3rd time step.

c

c       II.	Line-Source; Gaussian pulse: PEC square cylinder (tm_box)

c  The second animation-visualization program is that of a line-source

c  excited by a single Gaussian pulse radiating inside a Perfectly Electric

c  Conducting (PEC) square cylinder, using the Finite-Difference

c  Time-Domain method.  The MATLAB m-file  produces the FDTD solution

c  of an infinite length line-source excited by a time-derivative Gaussian

c  pulse in a 2-D TMz computational domain.  The m-file produces a movie

c  which is 70 frames long by taking a picture of the computational domain

c  every 3rd time step.

c

c      III.	E-Plane sectoral horn: unbounded medium (te_horn)

c  The third animation-visualization program is that of an E-plane sectoral

c  (2-D) horn antenna radiating into an unbounded medium, using the

c  Finite-Difference Time-Tomain method.  The unbounded medium is

c  simulated using a Berenger Perfectly-Matched Layer (PML) Absorbing

c  Boundary Condition (ABC) in order to truncate the computational

c  domain.  The MATLAB m-file produces the FDTD solution of the E-plane

c  sectoral (2-D) horn antenna excited by a sinusoidal voltage in

c  a TEz computational domain.  The m-file produces a movie which is 70

c  frames long by taking a picture of the computational domain every 3rd

c  time step.

c

c  **Note:

c  In order to animate and then visualize these three radiation problems, the

c  user needs an edition of MATLAB and the MATLAB m-file  found in the

c  included computer disc to produce the corresponding FDTD solution of

c  each radiation problem.  Additional details on the use of each

c  visualization problem are found in the computer disc included with this

c  book.

c************************************************************